Application Note

Modbus TCP CMMTt111.dll

This document describe how to use CMMTt111.dll to establish CMMT-AS/ST-...-MP Modbus TCP communication between PC and CMMT-XX-EP/MP with visual studio sample application.

100314

Title: Modbus TCP CMMTt111.dll

Version: v1.10

Document no.: 100314

Original: En

Author: Festo

Last saved: 15.07.2026

Copyright Notice

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files, to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to persons to whom the Software is furnished to do so, subject to the following conditions: The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

Legal Notice

Hardware, software, operating systems and drivers may only be used for the applications described and only in conjunction with components recommended by Festo SE & Co. KG.

Festo SE & Co. KG does not accept any liability for damages arising from the use of any incorrect or incomplete information contained in this documentation or any information missing therefrom.

Defects resulting from the improper handling of devices and modules are excluded from the warranty.  The data and information specified in this document should not be used for the implementation of safety functions relating to the protection of personnel and machinery.

No liability is accepted for claims for damages arising from a failure or functional defect. In other respects, the regulations with regard to liability from the terms and conditions of delivery, payment and use of software of Festo SE & Co. KG, which can be found at www.festo.com  and can be supplied on request, shall apply. All data contained in this document do not represent guaranteed specifications, particularly with regard to functionality, condition or quality, in the legal sense.

The information in this document serves only as basic information for the implementation of a specific, hypothetical application and is in no way intended as a substitute for the operating instructions of the respective manufacturers and the design and testing of the respective application by the user.

The operating instructions for Festo products can be found at www.festo.com .

Users of this document (application note) must verify that all functions described here also work correctly in the application. By reading this document and adhering to the specifications contained therein, users are also solely responsible for their own application.

© (Festo SE & CO. KG, D-73726 Esslingen, 2022)

Internet: 	http://www.festo.com

Table of contents

1	Important instruction	5

1.1	Designated use	5

1.2	Components and Software used for test	5

1.3	Topology of the tested system	5

1.4	Recommended manual	5

2	Overview	6

2.1	Process data	6

2.2	Telegram 111	7

2.3	Festo Automation Suite	7

2.3.1	New Project	7

2.3.2	Start first setup	8

2.3.3	Fieldbus	10

2.3.4	Record Table	12

2.3.5	Extended process data	12

3	CMMTt111.dll	13

3.1	Method	13

3.2	Properties	14

4	Implementation example in Visual Studio Community	21

4.1	Initialisation	21

4.2	Essential parameters	23

4.3	Connect	23

4.4	Gain control	23

4.5	Automatic Control	24

4.6	Jog Control	25

4.7	STW1, POS_STW1 and POS_STW2 direct access	25

Important instruction

# Important instruction

## Designated use

This application note describes how to use CMMTt111.dll. It is designed to establish Modbus TCP communication between PC and Festo motor controller CMMT-AS-…-EP and CMMT-ST-…-EP based on telegram 111 drive profile.

## Components and Software used for test

| Type/Name | Version Software/Firmware |
| --- | --- |
| Festo Automation Suite | V2.11.0.657 |
| CMMT-AS-C2-3A-MP-S1 | V36.12.0 |
| CMMT-AS-C2-3A-MP-S3 | V36.12.0 |
| CMMT-ST-C8-1C-MP-S0 | V35.12.0 |
| CMMT-AS Plug-In | 2.11.1.7 |
| CMMT-ST Plug-In | 2.11.1.7 |
| Operating System | Windows 11 |
| CMMTt111.dll | V1.10 |

Table 1.2:  Components/Software used

## Topology of the tested system

Figure 1.3: Topology of the tested system

## Recommended manual

Recommended manual can be found via this link:

https://www.festo.com/assets/attachment/de/665942

Alternatively, it can be searched via Festo Online eXperience (FoX) and search for “CMMT-AS”. Link:

https://www.festo.com/my/en/a/8143164/?q=CMMT-AS-C4-3A~:festoSortOrderScored

Figure 1.4: Recommended manual

# Overview

## Process data

The process data for each connection consists of the following:

- 12 Word (24 bytes) of Telegram 111 (PROFIdrive over Modbus TCP)

- Up to 16 Word (32 bytes) of extended process data (FAS feature - optional)

Figure 2.1: Process Data

## Telegram 111

| Word | Set point | Modbus Address |  | Actual value | Modbus Address |
| --- | --- | --- | --- | --- | --- |
| 1 | STW1 | 40001 |  | ZSW1 | 30001 |
| 2 | POS_STW1 | 40002 |  | POS_ZSW1 | 30002 |
| 3 | POS_STW2 | 40003 |  | POS_ZSW2 | 30003 |
| 4 | STW2 | 40004 |  | ZSW2 | 30004 |
| 5 | OVERRIDE | 40005 |  | MELDW | 30005 |
| 6 | MDI_TARPOS1 | 40006 |  | XIST_A1 | 30006 |
| 7 | MDI_TARPOS2 | 40007 |  | XIST_A2 | 30007 |
| 8 | MDI_VELOCITY1 | 40008 |  | NIST_B1 | 30008 |
| 9 | MDI_VELOCITY2 | 40009 |  | NIST_B2 | 30009 |
| 10 | MDI_ACC | 40010 |  | FAULT_CODE | 30010 |
| 11 | MDI_DEC | 40011 |  | WARN_CODE | 30011 |
| 12 | Reserve | 40012 |  | Reserve | 30012 |

Function code 4: Read Input Register             – Read actual value from CMMT

Function code 16: Write Multiple Register     – Write set point to CMMT

Table 2.2:  Telegram 111 in Modbus TCP

## Festo Automation Suite

Important setting in Festo Automation Suite to establish Modbus TCP communication between PC and CMMT.

### New Project

Open FAS, create new project:

Figure 2.3.1.1: Create New Project

Check FAS version 	Check Plug in version:

Figure 2.3.1.2: Check FAS version 	 	Figure 2.3.1.3: Check Plug in version

Add new CMMT: 	 Open CMMT Plug In:

Figure 2.3.1.4: Add new CMMT 	 Figure 2.3.1.5: Open CMMT Plug In

### Start first setup

Figure 2.3.2.1: Start first setup

Similar step to add Axis and Mounting Kit:

Figure 2.3.2.2: Complete device configuration

Click on “Next” to continue to the next page. On the device setting, select activation via “Fieldbus (2)”.

Figure 2.3.2.3: Device settings

On fieldbus configuration, select “Ethernet/IP – ModbusTCP (3)”. On telegram selection, select “Telegram 111” and insert IP Address for Modbus Communication.

Change as you need in the next setup. Once finish, click on finish.

Figure 2.3.2.4: Finish

### Fieldbus

Factor group setting is related to Table 3.7: Group Setting, ConvertPositionInput and ConvertPositionOutput

Figure 2.3.3.1: Factor group

This setting is related to Table 3.16: Group output – OutW [] Processed, 100% setting Acceleration and Deceleration resulted value set here. “Base value velocity” value can be used as reference to set ConvertVelocityOutput, see figure 4.1.3.

Figure 2.3.3.2: Base value

### Record Table

Create record list as below:

Figure 2.3.4: Record table

### Extended process data

Figure below shows how to send and receive record table field 2 (Target position) via extended process data.

Figure 2.3.5.1: Get parameter number from record table

Figure 2.3.5.2: Adding extended process data

The total number of bytes for both send and receive data are 8 bytes which is equivalent with 4 words.

# CMMTt111.dll

## Method

| Method |  |
| --- | --- |
| Namespace | CMMTt111 |
| Class | festoCMMT |

Table 3.1:  Method

To connect to CMMT via Modbus TCP

| Constructor | Connect(string ipAddress, byte IW, byte OW) |
| --- | --- |
| ipAddress | IP Address of CMMT-EP |
| IW | Number of extended process data input in word (16 bit) |
| OW | Number of extended process data output in word (16 bit) |
| Port | Default = 502, cannot be changed |

Table 3.2:  Connect

To disconnect from CMMT

#### Constructor 	Disconnect()

| Table 3.3: Disconnect To end thread in CMMTt111.dlll. Must be called | when exit application |
| --- | --- |
| Constructor | ExitApplication() |

Table 3.4:  Disconnect

Automatic control function

| String | CmmtControl(byte mode, byte cmd) |
| --- | --- |
| mode | 1: homing mode |
|  | 2: absolute positioning mode |
|  | 3: relative positioning mode |
|  | 4: record positioning mode |
| cmd | 0 --> 1 : execute mode |
|  | 0 --> 2 : stop movement (halt) |
|  | 0 --> 3 : clear remaining position |
|  | 0 --> 4 : acknowledge malfunction and set output stage |
| String response: |  |
| No Output stage enable | !OutputStageEnable & active Cmd != 4 |
| Malfunction, fault/warning: <Fault / Warn code> | Malfunction Effective |
| Unable to reset malfunction | Timeout during cmd=4 |
| Set Output Stage.. | OutputStageEnable: 0 --> 1 |
| Operation enabled | OperationEnabled |
| Can't enable operation | Timeout during enabling operation |
| Operation is not enabled | !OperationEnabled |
| homing... | Homing start, mode=1, cmd=1 |
| homing time-out | Homing timeout during homing mode |
| Homing done | Homing success |
| Homing interrupted | Stop triggered during homing |
| moving... | Move absolute, mode=2 or mode=3, cmd=1 |
| time-out | Time out during move absolute/relative/record position |
| Position reached | Move absolute/relative done |
| Position record selection is not set | PositionRecord == 0 |
| moving to record selection...X | Move record position, mode=4, cmd=1 |
| Position reached: X | Move record position done |
| Intermediate Stop executed | cmd=2, IntermediateStop=0 |
| Remaining position cleared | IntermediateStop==0 & cmd=3, RejectPositioningTask=0 |

String response in integer --> check properties icmmtStatus

Table 3.5:  CmmtControl

## Properties

| Properties |  |
| --- | --- |
| Namespace | CMMTt111 |
| Class | festoCMMT |

Table 3.6:  Properties

| Group setting |  | {Get; Set;} |
| --- | --- | --- |
| float | ConvertPositionInput | ActualPosition = (XIST_A1 + XIST_A2 * 65536) / ConvertPositionInput |
| float | ConvertVelocityInput | ActualVelocity = (NIST_B1 + NIST_B2 * 65536) / ConvertVelocityInput |
| float | ConvertPositionOutput | MDI_TarPos1 = (TargetPosition * ConvertPositionOutput) % 65536 |
|  |  | MDI_TarPos2 = (TargetPosition * ConvertPositionOutput) / 65536; |
| float | ConvertVelocityOutput | MDI_Velocity1 = (TargetVelocity * ConvertVelocityOutput) % 65536 |
|  |  | MDI_Velocity2 = (TargetVelocity * ConvertVelocityOutput) / 65536; |
| int | CycleTime = 15 | Cycle time to read and write to CMMT via modbus, 10 = 10 ms |
| Int | ModbusTimeOut | Modbus time out, 0 ms – 10000 ms |
| int | Timeout = 0 | Time out during cmd =1 in CMMTControl |
|  |  | Timeout = 20000 * CycleTime = 200000ms = 200 second |
| int | Timewait = 5 | Time wait during cmd =1 in CMMTControl |
| Group setting |  | {Get; Set;} |
|  |  | Timewait = 5 * CycleTime = 50ms |
| Int | Homingtimeout = 0 | Homing time out during mode=1, cmd=1 in CMMTControl |
|  |  | Homingtimeout = 1000 * CycleTime = 10000ms = 10 second |
| bool | AccessOutputWord_STW1 = false; | True = mentioned Output Word can be modified from code False = mentioned Output Word will follow Output BIT |
| bool | AccessOutputWord_POS_STW1 = false; |  |
| bool | AccessOutputWord_POS_STW2 = false; |  |
| Int | SetReadTimeOut | Default = 30ms, minimum 10ms, skip read data from CMMT when time out reached |
| Int | MaxReadTime | Keep the largest ActualReadTime (ms) |
| Int | MaxReadmbTime | Keep the largest ActualReadmbTime (ms) |
| Int | MaxWritembTime | Keep the largest ActualWritembTime (ms) |
| Int | CounterPass | Count number of ActualReadTime that exceed SetReadTimeOut |

Table 3.7:  Group Setting

| Group status |  | {Get;} |
| --- | --- | --- |
| string | Version | = current version and release date |
| bool | Connected = false; | = True, when connected to CMMT |
| bool | ConnectionError = false; | = True, if connection disturbed, Connected will still = true |
| int | iRetryConnection = 0; | increase every cycle time when ConnectionError = true |
| string | StatusRead = ""; | String response of reading actual value from CMMT |
| string | StatusWrite = ""; | String response of writing set point to CMMT |
| string | Status = ""; | connection status in string |
|  |  | = Connection retry + iRetryConnection |
|  |  | = Disconnected |
|  |  | = Retry in 5 sec --> failed more than 3 times with cycle time, retry in 5 sec |
|  |  | = Connected |
|  |  | = Can't connect --> check cable, IP address |
|  |  | = Conversion failed --> check ConvertPositionInput, etc, Overflow? |
| Int | iCmmtStatus | = integer response of CmmtControl method |
|  | = 1 | CmmtStatus = "No Output stage enable" |
|  | = 2 | CmmtStatus = "Malfunction, fault/warning |
|  | = 405 | CmmtStatus = "Unable to reset malfunction" |
|  | = 4010 | CmmtStatus = "Set Output Stage.." |
|  | = 40301 | CmmtStatus = "Operation enabled" |
|  | = 40302 | CmmtStatus = "Can't enable operation" |
|  | = 1101 | CmmtStatus = "Operation is not enabled" |
|  | = 1102 | CmmtStatus = "homing..." |
|  | = 11201 | CmmtStatus = "homing time-out" |
|  | = 11202 | CmmtStatus = "Homing done" |
|  | = 11203 | CmmtStatus = “Homing interrupted” |
|  | = 150 | CmmtStatus = "Operation is not enabled" |
|  | = 1510 | CmmtStatus = "time-out" |
|  | = 15301 | CmmtStatus = "time-out" |
|  | = 15302 | CmmtStatus = "Position reached" |
|  | = 15303 | CmmtStatus = "moving..." |
|  | = 1401 | CmmtStatus = "Operation is not enabled" |
|  | = 1402 | CmmtStatus = "Position record selection is not set" |
|  | = 1410 | CmmtStatus = "time-out" |
|  | = 14301 | CmmtStatus = "time-out" |
|  | = 14302 | CmmtStatus = "Selection reached: " |
|  | = 14303 | CmmtStatus = "moving to record selection..." |
|  | = 21 | CmmtStatus = "Intermediate Stop executed" |
|  | = 31 | CmmtStatus = "Remaining position cleared" |
| Int | ActualReadTime | To monitor actual read time (ms) |
| Int | ActualReadmbTime | To monitor actual read cycle time (ms) |
| Group status |  | {Get;} |
| Int | ActualWritembTime | To monitor actual write cycle time (ms) |

Table 3.8:  Group Status

| Group input – InW [] |  | {Get;} |
| --- | --- | --- |
| int | ZSW1 | = InW[0] |
| int | POS_ZSW1 | = InW[1] |
| int | POS_ZSW2 | = InW[2] |
| int | ZSW2 | = InW[3] |
| int | MELDW | = InW[4] |
| int | XIST_A1 | = InW[5] |
| int | XIST_A2 | = InW[6] |
| int | NIST_B1 | = InW[7] |
| int | NIST_B2 | = InW[8] |
| int | Fault_Code | = InW[9] |
| int | Warn_Code | = InW[10] |
| int |  | = InW[11] |

Table 3.9:  Group input – InW []

| Group input – InW [] Processed |  | {Get;} |
| --- | --- | --- |
| float | ActualPosition | ➔ properties group setting ConvertPositionInput |
| float | ActualVelocity | ➔ properties group setting ConvertVelocityInput |

Table 3.10:  	Group input – InW [] Processed

| Group input bit – ZSW1 |  | {Get;} |
| --- | --- | --- |
| bool | ReadytobeSwitchOn | = InW[0].0 |
| bool | ReadyforOperation | = InW[0].1 |
| bool | OperationEnabled | = InW[0].2 |
| bool | MalfunctionEffective | = InW[0].3 |
| bool | CoastingActive (NC) | = InW[0].4 |
| bool | FastStopActive (NC) | = InW[0].5 |
| bool | SwitchOnLockActive | = InW[0].6 |
| bool | WarningEffective | = InW[0].7 |
| bool | PositionInRange | = InW[0].8 |
| bool | GuideRequired | = InW[0].9 |
| bool | ReadyforOperation | = InW[0].10 |
| bool | ReferencePointSet | = InW[0].11 |
| bool | PositioningTaskActivated | = InW[0].12 |
| bool | DriveStationary | = InW[0].13 |
| bool | AxisAccelerated | = InW[0].14 |
| bool | DriveDecelerated | = InW[0].15 |

Table 3.11:  	Group Input bit – ZSW1

| Group input bit – POS_ZSW1 |  | {Get;} |
| --- | --- | --- |
| bool | ActivePositioningRecordBit0 | = InW[1].0 |
| bool | ActivePositioningRecordBit1 | = InW[1].1 |
| bool | ActivePositioningRecordBit2 | = InW[1].2 |
| bool | ActivePositioningRecordBit3 | = InW[1].3 |
| bool | ActivePositioningRecordBit4 | = InW[1].4 |
| bool | ActivePositioningRecordBit5 | = InW[1].5 |
| bool | ActivePositioningRecordBit6 | = InW[1].6 |
| bool |  | = InW[1].7 |
| Group input bit – POS_ZSW1 |  | {Get;} |
| bool | NegativeLimitSwitchActive | = InW[1].8 |
| bool | PositiveLimitSwitchActive | = InW[1].9 |
| bool | JoggingActive | = InW[1].10 |
| bool | HomingActive | = InW[1].11 |
| bool |  | = InW[1].12 |
| bool | PositioningRecordActive | = InW[1].13 |
| bool |  | = InW[1].14 |
| bool | MDIActive | = InW[1].15 |

Table 3.13:  	Group Input bit – POS_ZSW1

| Group input bit – POS_ZSW1 Processed |  | {Get;} |
| --- | --- | --- |
| byte | ActivePositioningRecord | Active positioning record in byte |

Table 3.14:  	Group Input bit – POS_ZSW1 Processed

| Group input bit – POS_ZSW2 |  | {Get;} |
| --- | --- | --- |
| bool | TrackingModeActive | = InW[2].0 |
| bool | VelocityLimitingActive | = InW[2].1 |
| bool | SetPointValueStopped | = InW[2].2 |
| bool |  | = InW[2].3 |
| bool | DriveTravelsForward | = InW[2].4 |
| bool | DriveTravelsBackward | = InW[2].5 |
| bool | NegativeSoftwareLimitSwitchActive | = InW[2].6 |
| bool | PositiveSoftwareLimitSwitchActive | = InW[2].7 |
| bool | ActualPositionLessEqualThanCamSwitch0 | = InW[2].8 |
| bool | ActualPositionLessEqualThanCamSwitch1 | = InW[2].9 |
| bool | DirectOutput1ViaPositioningRecord | = InW[2].10 |
| bool | DirectOutput2ViaPositioningRecord | = InW[2].11 |
| bool | FixedStopReached | = InW[2].12 |
| bool | FixedStopClampingTorqueReached | = InW[2].13 |
| bool | TravelToFixedStopActive | = InW[2].14 |
| bool | PositionCommandActive | = InW[2].15 |

Table 3.15:  	Group Input bit – POS_ZSW1

| Group output – OutW [] |  | {Get; Set;} |
| --- | --- | --- |
| int | STW1 | OutW[0] = STW1 = Group output bit STW1 ➔ Properties group setting, AccessOutputWord_STW1 = False |
| int | POS_STW1 | OutW[1] = POS_STW1 = Group output bit POS_STW1 ➔ Properties group setting, AccessOutputWord_POS_STW1 = False |
| int | POS_STW2 | OutW[2] = POS_STW2 = Group output bit POS_STW2 ➔ Properties group setting, AccessOutputWord_POS_STW2 = False |
| int | STW2 | OutW[3] = STW2 |
| int | iOverride | OutW[4] = iOverride |
| int | MDI_TarPos1 | OutW[5] = MDI_TarPos1 |
| int | MDI_TarPos2 | OutW[6] = MDI_TarPos2 |
| int | MDI_Velocity1 | OutW[7] = MDI_Velocity1 |
| int | MDI_Velocity2 | OutW[8] = MDI_Velocity2 |
| int | MDI_ACC | OutW[9] = MDI_ACC |
| Group output – OutW [] |  | {Get; Set;} |
| int | MDI_DEC | OutW[10] = MDI_DEC |
| int |  | OutW[11] |

Table 3.16:  	Group output – OutW []

| Group output – OutW [] Processed |  | {Get; Set;} |
| --- | --- | --- |
| float | Override | iOverride = (Override * 16384) / 100 |
| float | TargetPosition | MDI_TarPos1 = (TargetPosition * ConvertPositionOutput) % 65536 MDI_TarPos2 = (TargetPosition * ConvertPositionOutput) / 65536 |
| float | TargetVelocity | MDI_Velocity1 = (TargetVelocity * ConvertVelocityOutput) % 65536 MDI_Velocity2 = (TargetVelocity * ConvertVelocityOutput) / 65536 |
| float | Acceleration | MDI_ACC = (Acceleration * 16384) / 100 |
| float | Deceleration | MDI_DEC = (Deceleration * 16384) / 100; |

Table 3.17:  	Group output – OutW [] Processed

| Group output bit – STW1 |  | {Get; Set;} |
| --- | --- | --- |
| bool | OutputStageEnable | OutW[0].0 |
| bool | DriveCoast | OutW[0].1 |
| bool | FastStop | OutW[0].2 |
| bool | EnableOperation | OutW[0].3 |
| bool | RejectPositioningTask (NC) | OutW[0].4 |
| bool | IntermediateStop (NC) | OutW[0].5 |
| bool | ActivePositioningTask | OutW[0].6 |
| bool | AcknowledgeMalfunction | OutW[0].7 |
| bool | Jogging1 | OutW[0].8 |
| bool | Jogging2 | OutW[0].9 |
| bool | PLCMastedControl | OutW[0].10 |
| bool | StartHoming | OutW[0].11 |
| bool | ReleaseHonfingBrake | OutW[0].12 |
| bool |  | OutW[0].13 |
| bool |  | OutW[0].14 |
| bool |  | OutW[0].15 |

Table 3.18:  	Group output bit – STW1

| Group output bit – POS_STW1 |  | {Get; Set;} |
| --- | --- | --- |
| bool | PositioningRecordSelectionBit0 | OutW[1].0 |
| bool | PositioningRecordSelectionBit1 | OutW[1].1 |
| bool | PositioningRecordSelectionBit2 | OutW[1].2 |
| bool | PositioningRecordSelectionBit3 | OutW[1].3 |
| bool | PositioningRecordSelectionBit4 | OutW[1].4 |
| bool | PositioningRecordSelectionBit5 | OutW[1].5 |
| bool |  | OutW[1].6 |
| bool | RelativePositioning | OutW[1].7 = 0 |
| bool | AbsolutePositioning | OutW[1].7 = 1 |
| bool |  | OutW[1].8 |
| bool |  | OutW[1].9 |
| bool |  | OutW[1].10 |
| bool |  | OutW[1].11 |
| bool |  | OutW[1].12 |
| bool |  | OutW[1].13 |
| bool |  | OutW[1].14 |
| bool | MDISelection | OutW[1].15 |

Table 3.20:  	Group output bit – POS_STW1

| Group output bit – POS_STW2 |  | {Get; Set;} |
| --- | --- | --- |
| bool | TrackingMode | OutW[2].0 |
| bool | CurrentPositionasHome | OutW[2].1 |
| bool |  | OutW[2].2 |
| bool |  | OutW[2].3 |
| bool |  | OutW[2].4 |
| bool | IncrementalJogging | OutW[2].5 |
| bool |  | OutW[2].6 |
| bool |  | OutW[2].7 |
| bool |  | OutW[2].8 |
| bool |  | OutW[2].9 |
| bool | TouchProbeSource | OutW[2].10 |
| bool | TouchProbeEdge | OutW[2].11 |
| bool |  | OutW[2].12 |
| bool |  | OutW[2].13 |
| bool | ActivateSoftwareLimitSwitch | OutW[2].14 |
| bool | ActivateHardwareLimitSwitch | OutW[2].15 |

Table 3.21:  	Group output bit – POS_STW2

# Implementation example in Visual Studio Community

## Initialisation

Add “using CMMTt111”

Figure 4.1.1: using CMMTt111

Declare class festoCMMT:

festoCMMT = new festoCMMT();

Implementation example:

Implementation example in Visual Studio Community

Figure 4.1.2: Implementation example

How to set position and velocity input output is by clicking on the “?” button to open help menu. Fill up the FAS factor Group and FAS Reference values (Section 2.3.4 - Fieldbus) as shown below. The value in “Factor Group” can be copy and paste manually from help menu to main interface.

Figure 4.1.3: Setting factor group

## Essential parameters

Before Connect, it is essential to set below parameters:

// essential setting        // cycle time

festoCMMT.CycleTime = 15; // 15 ms

// modbus time out        festoCMMT.ModbusTimeOut = 1000; // 500 ms

// read time out

festoCMMT.ReadTimeOut = 100; // 100 ms   // Conversion & dynamic - See page 9 ------------------------------------------

// Conversion

| festoCMMT.ConvertPositionOutput = 1000000; | // Factor group, position = -6 |
| --- | --- |
| festoCMMT.ConvertVelocityOutput = 1000; | // Factor group, velocity = -3 |
| festoCMMT.ConvertPositionInput = 1000000; | // Factor group, position = -6 |
| festoCMMT.ConvertVelocityInput = 214748364; | // Reference value, base value |
|  | // velocity = 5 |

// dynamic

festoCMMT.Acceleration = 100; // 100% of reference value, base value acceleration        festoCMMT.Deceleration = 100; // 100% of reverence value, base value deceleration

// ----------------------------------------------------------------------------

// time out

festoCMMT.Timeout = 0; // no time out, Command abort when reach time out        festoCMMT.Homingtimeout = 1000; // 1000 x cycle time = 20 sec homing time out        festoCMMT.Timewait = 5; //5 * cycle time = 100ms, Wait before check next condition

// Activate software and hardware limit        festoCMMT.ActivateHardwareLimitSwitch = true;        festoCMMT.ActivateSoftwareLimitSwitch = true;

## Connect

Command to establish communication between PC and CMMT:

string IPAddress  = “192.168.2.44” // example IP Address

int ExtendedIW  	= 4 // (Word) Extended process data in – int 16 bit

int ExtendedOW  	= 4 // (Word) Extended process data out – int 16 bit

ExtendedIW and ExtendedOW above are set based on setting in chapter 2.3.5 Extended process data.

festoCMMT.Connect(IPAddress, ExtendedIW,ExtendedOW);

Response:

festoCMMT.Connected

= True  --> Connected

= False  --> Not connected

## Gain control

To gain control from PC ad obtain ready to be switched on:

festoCMMT.PLCMasterControl = true;

festoCMMT.EnableOperation = true;        festoCMMT.FastStop = true;

festoCMMT.DriveCoast = true;

Response:

festoCMMT.ReadytobeSwitchedOn == true;

## Automatic Control

Parameter to set before automatic control:

// override      festoCMMT.Override = 100.0F; // 100%, 0% to 199%

// Target

festoCMMT.TargetPosition = 10.5F; // in user unit, ex: 10.5 rev         festoCMMT.TargetVelocity = 5.0F; // in user unit, ex: 5.0 rev/sec

// Record position       festoCMMT.PositioningRecord = 1; // FAS record list = 1

CmmtControl:

// Choose 1 Mode

Mode = 1; 	// Homing mode

// Mode = 2;  // Absolute positioning mode

// Mode = 3;  // Relative positioning mode

// Mode = 4;  // Record positioning mode

Implementation example in Visual Studio 2019

// Choose 1 Cmd

| // Cmd = 1; | // Execute Mode |
| --- | --- |
| // Cmd = 2; | // Stop movement (halt) |
| // Cmd = 3; | // Clear remaining position |
| Cmd = 4; | // Acknowledge malfunction and set output stage |

festoCMMT.CmmtControl(Mode, Cmd); // Define Mode and Cmd before execute

Cmd = 0; // Immediately set Cmd to 0

Example sequence:

- Acknowledge malfunction and set output stage o Mode = X  Mode = 1 o Cmd = 4  Cmd = 4

- Obtain homing

  - Mode = 1 	 	Mode = 1 o 	Cmd = 1 	 	Cmd = 1

- Absolute positioning mode

  - Set override 	 	festoCMMT.Override o 	Set Target position 	festoCMMT.TargetPosition o 	Set Target velocity 	festoCMMT.TargetVelocity

  - Mode = 2 	 	Mode = 2 o 	Cmd = 1 	 	Cmd = 1

- Relative positioning mode

  - Set override 	 	festoCMMT.Override o 	Set Target position 	festoCMMT.TargetPosition o 	Set Target velocity 	festoCMMT.TargetVelocity

  - Mode = 3 	 	Mode = 3

  - Cmd = 1 	 	Cmd = 1

- Stop during move relative

  - Mode = X 	 	Mode = 3

  - Cmd = 2 	 	Cmd = 2

- Continue relative positioning mode

|  | o Mode = 3 | Mode = 3 |
| --- | --- | --- |
|  | o Cmd = 1 | Cmd = 1 |
| • | Stop during move relative |  |
|  | o Mode = X | Mode = 3 |
|  | o Cmd = 2 | Cmd = 2 |
| • | Clear remaining position |  |
|  | o Mode = X | Mode = 3 |
|  | o Cmd = 3 | Cmd = 3 |
| • | Record positioning mode |  |
|  | o Set override | festoCMMT.Override |
|  | o Set Positioning record | festoCMMT.PositioningRecord |
|  | o Mode = 4 | Mode = 4 |
|  | o Cmd = 1 | Cmd = 1 |

Actual position and actual velocity can be monitored (Table 3.10: Group input – InW [] Processed):

festoCMMT.ActualPosition festoCMMT.ActualVelocity

Status:

String feedback from festoCMMT.CmmtControl can be seen in Table 3.5 CmmtControl (String response). Integer feedback can be seen in Table 3.8 Group status (iCmmtStatus).

## Jog Control

To Jog, homing must be obtained by observing parameter  festoCMMT.ReferencePointSet (== true is homed/referenced).

Example:

Jog- :

festoCMMT.Jog1 = true;

festoCMMT.Jog2 = false;

Jog+ :

festoCMMT.Jog2 = true;

festoCMMT.Jog1 = false;

Response while jog- or jog+ is true:

festoCMMT.JoggingActive == true;

## STW1, POS_STW1 and POS_STW2 direct access

Output STW1, POS_STW1 and POS_STW2 are manipulated by accessing Output bit listed in Table 3.18 – Table 3.21. Those output can also be access directly by activating these parameter:

festoCMMT.AccessOutputWord_STW1		= true;

festoCMMT.AccessOutputWord_POS_STW1	= true;

festoCMMT.AccessOutputWord_POS_STW2	= true;

After enabling above parameters, STW1, POS_STW1 and POS_STW2 listed in Group Output, Table 3.16, can be directly manipulated.

Example:

to set festoCMMT.PLCMasterControl = true; (OutW[0].10, Table 3.18 - Group output bit – STW1) Direct access:

festoCMMT.AccessOutputWord_STW1 = true;

festoCMMT.STW1 = 1024;